PicoRV32 SoC with AXI-Lite + UART — Complete Course Index	2

Course Overview	2

Quick Reference: Lab Commands	2

Sub-module Map	3

Sub-module Documentation	4

Project File Map	4

RTL Source (rtl/)	4

Testbenches (tb/)	4

Firmware (fw/)	5

Synthesis (openlane_design/)	5

Address Map — Definitive Reference	5

SoC Parameters — Quick Reference	5

AXI-Lite Signal Quick Reference	6

CPU-to-AXI Bridge FSM	6

Simulation Output Reference	6

Expected make soc Output	6

Expected make axi_sa Output	7

ASIC Design Notes	7

Interview Preparation Topics	7

RISC-V ISA	7

AXI-Lite	7

UART	8

SoC Integration	8

Firmware	8

Waveform Files Reference	8

# PicoRV32 SoC with AXI-Lite + UART — Complete Course Index

Course: Design and Implementation of a RISC-V Based SoC Subsystem
Technology: PicoRV32 (RV32I) + AXI-Lite + UART + Verilator + GTKWave + OpenLane

## Course Overview

This course takes you from zero to a complete, ASIC-ready SoC design. You will:

- Understand RISC-V ISA, AXI-Lite protocol, and UART communication

- Implement a full SoC subsystem in Verilog with 9 RTL modules

- Verify each component and the complete system using Verilator simulation

- Write embedded C firmware that runs on the CPU and communicates via UART

- Synthesize the design using Yosys + OpenLane2 for ASIC tape-out readiness

## Quick Reference: Lab Commands

| Sub-module | Topic | Command |
| --- | --- | --- |
| M1 | RISC-V ISA | make pico |
| M2 | AXI-Lite | make axi_sa |
| M3 | UART | make uart |
| M4 | MMI RISC-V + UART | make soc_pico_uart_mem |
| M5 | RISC-V + AXI | make riscv_axi |
| M6 | AXI + UART | make axi_uart |
| M7 | Firmware | make fw |
| M8 | Full SoC | make soc / make debug_soc |
| M9 | Waveform | make wave |
| — | Common | make clean / make synth |

## Sub-module Map

s

## Sub-module Documentation

| SN | Sub-module | Lab Document | Key Lab Command |
| --- | --- | --- | --- |
| 1 | Fundamentals of RISC-V ISA | Sub-module_1_lab.md | make pico |
| 2 | Introduction to AXI-Lite | Sub-module_2_lab.md | make axi_sa |
| 3 | UART Protocol | Sub-module_3_lab.md | make uart |
| 4 | MMI RISC-V + UART | Sub-module_4_lab.md | make soc_pico_uart_mem |
| 5 | RISC-V AXI Integration | Sub-module_5_lab.md | make riscv_axi |
| 6 | AXI + UART Integration | Sub-module_6_lab.md | make axi_uart |
| 7 | Firmware Implementation | Sub-module_7_lab.md | make fw |
| 8 | Full SoC Integration | Sub-module_8_lab.md | make soc |
| 9 | Waveform Analysis | Sub-module_9_lab.md | make wave |

## Project File Map

### RTL Source (rtl/)

| File | Sub-module | Role |
| --- | --- | --- |
| picorv32.v | M1/M8 | PicoRV32 CPU — full RV32I implementation |
| top.v | M5/M8 | SoC top: CPU + bridge + interconnect wiring |
| axi_lite_interconnect.v | M2/M8 | 1-master, 3-slave AXI-Lite router |
| axi_decoder.v | M2/M8 | Combinational one-hot address decode |
| rom.v | M2/M8 | AXI-Lite ROM (sim: readmemh, synth: case) |
| sram.v | M2/M8 | AXI-Lite SRAM with byte-enable write |
| uart_axi.v | M6/M8 | UART as AXI-Lite slave (TX buf + RX buf) |
| uart_tx.v | M3/M6 | 8N1 UART transmitter FSM |
| uart_rx.v | M3/M6 | 8N1 UART receiver FSM + 2-FF sync |
| rom.hex | M7/M8 | Firmware binary: generated by make fw |

### Testbenches (tb/)

| File | Sub-module | Tests |
| --- | --- | --- |
| tb_top.v | M8/M9 | Full SoC: UART monitor + 10 greetings + PING echo |
| tb_debug.v | M8/M9 | Verbose debug version of tb_top |
| tb_axi_lite_standalone.v | M2 | AXI-Lite handshake demo (self-contained) |
| tb_riscv_axi_lite.v | M5 | CPU-AXI integration demo (self-contained) |
| tb_axi_lite.v | M6 | AXI master + uart_axi DUT test |
| tb_axi_lite_master.v | M6 | Reusable AXI master task library |
| tb_uart_top.v | M3 | UART TX/RX loopback test |

### Firmware (fw/)

| File | Role |
| --- | --- |
| main.c | Application: print greeting ×10 + echo loop |
| crt0.S | Startup: stack setup + call main |
| link.ld | Memory map: ROM@0x0, SRAM@0x10000 |
| Makefile | compile → link → objcopy → bin2hex |
| bin2hex.py | Converts binary to 4-wide hex for readmemh |
| gen_rom_case.py | Generates synthesizable case-ROM from hex |
| firmware.elf | Pre-built (inspect with riscv32-…-objdump) |
| firmware.bin | Raw 167-byte binary |

### Synthesis (openlane_design/)

| Goal | Command |
| --- | --- |
| Yosys synthesis | make synth |
| OpenLane2 full flow | make openlane |
| Synthesis flag | -D SYNTHESIS activates case-ROM path |

## Address Map — Definitive Reference

| Address Range | Peripheral | Slave Index | Bus |
| --- | --- | --- | --- |
| 0x0000_0000 – 0x0000_FFFF | ROM (firmware) | S0 | AXI-Lite |
| 0x0001_0000 – 0x0001_FFFF | SRAM (data/stack) | S1 | AXI-Lite |
| 0x1000_0000 | UART TX register | S2 | AXI-Lite |
| 0x1000_0004 | UART RX register | S2 | AXI-Lite |
| 0x1000_0008 | UART STATUS register | S2 | AXI-Lite |

## SoC Parameters — Quick Reference

| Parameter | Value | Notes |
| --- | --- | --- |
| CPU | PicoRV32 | RV32I, no MUL/DIV/FPU |
| Clock | 50 MHz | CLK_PERIOD = 20 ns |
| Boot address | 0x0000_0000 | PROGADDR_RESET |
| Stack top | 0x0001_00FC | STACKADDR |
| ROM depth | 2048 words = 8 KB | MEM_DEPTH=2048 |
| SRAM depth | 64 words = 256 B | MEM_DEPTH=64 |
| UART baud | 115200 | BAUD_RATE |
| CLKS_PER_BIT | 434 | 50 MHz / 115200 |
| UART bit period | 8.68 µs | 1/115200 |
| UART byte + frame | 86.8 µs | 10 bit-periods |

## AXI-Lite Signal Quick Reference

| Channel | Master drives | Slave drives | Direction |
| --- | --- | --- | --- |
| Write Address (AW) | AWADDR, AWVALID | AWREADY | M→S |
| Write Data (W) | WDATA, WSTRB, WVALID | WREADY | M→S |
| Write Response (B) | BREADY | BRESP, BVALID | S→M |
| Read Address (AR) | ARADDR, ARVALID | ARREADY | M→S |
| Read Data (R) | RREADY | RDATA, RRESP, RVALID | S→M |

Handshake rule: Transfer occurs at rising clock edge when VALID AND READY = 1.

## CPU-to-AXI Bridge FSM

| State | Value | Action |
| --- | --- | --- |
| ST_IDLE | 3’d0 | Wait for cpu_mem_valid |
| ST_WR_AW | 3’d1 | Issue AW+W, wait for AW/W handshakes + B response |
| ST_WR_B | 3’d2 | Wait for B channel (if AW+W already done) |
| ST_RD_AR | 3’d3 | Issue AR channel, wait for arready |
| ST_RD_R | 3’d4 | Wait for R data, latch into rdata_r |

## Simulation Output Reference

### Expected make soc Output

--------------------------------------------------
 UART SoC Simulation Started
--------------------------------------------------
[ROM] loaded from plusarg: rtl/rom.hex
[TB] UART Monitor Started
[TB] RX byte: H  ← Character-by-character from UART TX
...
[TB] Greeting message count = 10
============================================
 10 GREETING MESSAGES RECEIVED
 UART OUTPUT VERIFIED
============================================
[TB] FIFO flushed - sending command : PING
[TB] RX byte: P
[TB] RX byte: I
[TB] RX byte: N
[TB] RX byte: G
============================================
 TEST PASSED
 Echo received : PING
============================================

### Expected make axi_sa Output

======================================================
  AXI-Lite Standalone Testbench  (32-bit addr/data)
======================================================
[CHECK] Write Data == Read Data → PASS ✓
  SUMMARY :  Passed = 1   Failed = 0
======================================================

## ASIC Design Notes

| Category | Status |
| --- | --- |
| initial blocks in RTL | ✅ None (only in testbenches) |
| Combinational loops | ✅ None |
| Inferred latches | ✅ All outputs have defined defaults |
| synthesis path | ✅ Case-ROM in rom.v via -D SYNTHESIS |
| Yosys synthesis | ✅ make synth runs without fatal errors |
| OpenLane2 | ✅ make openlane (config in openlane_design/) |

## Interview Preparation Topics

### RISC-V ISA

- What are the 6 RISC-V instruction formats?

- How does the PicoRV32 memory interface work?

- What is the boot sequence of a RISC-V bare-metal system?

### AXI-Lite

- Explain the valid/ready handshake with an example

- Why are AW and W channels separate?

- What does WSTRB = 4’h1 mean?

- How does a slave signal backpressure on the write channel?

### UART

- Calculate CLKS_PER_BIT for 9600 baud at 25 MHz

- What is the purpose of the 2-FF synchronizer in uart_rx?

- Why sample at the middle of the bit period, not the edge?

### SoC Integration

- How does the CPU reading ROM differ from writing to UART at the protocol level?

- What keeps the AXI bridge in ST_WR_AW if the B channel never responds?

- What happens if two masters try to access the AXI slave simultaneously?

- How does a synthesizable ROM differ from an SRAM in RTL?

### Firmware

- What does volatile do in embedded C?

- Why is -nostartfiles used in the firmware compilation?

- What does crt0.S do before calling main()?

## Waveform Files Reference

| VCD/FST File | From | Size | Contents |
| --- | --- | --- | --- |
| tb_top.fst | make soc | ~5 MB | Full SoC simulation |
| tb_top_debug.fst | make debug_soc | ~5 MB | Debug TB version |
| tb_axi_lite.vcd | make axi_uart | 1.9 MB | AXI-UART module |
| tb_axi_lite_standalone.vcd | make axi_sa | 3.8 KB | AXI handshake demo |
| tb_riscv_axi_lite.vcd | make riscv_axi | 3.3 KB | CPU-AXI demo |
| tb_uart.vcd | make uart | 2.9 MB | UART loopback |
| tb_picorv32.vcd | make pico | 36 KB | PicoRV32 standalone |
| tb_mem_soc.vcd | make soc_pico_uart_mem | 142 MB | MMI SoC (direct bus) |