Fundamentals of RISC-V ISA	1

1. Title	1

2. Objective	2

3. Theory (Multi-Level)	2

3.1 Beginner — Simple Intuition	2

3.2 Intermediate — Engineering View	2

3.3 Advanced — Signal/Architecture Level	2

4. Architecture / Concept	3

PicoRV32 CPU Block Diagram	3

Instruction Flow (Fetch → Decode → Execute → Memory → Writeback)	4

5. Implementation	4

Files in This Module	4

How It Works in This Project	4

6. Lab / Hands-on Section	4

Prerequisites	4

Step-by-Step Commands	4

7. Waveform / Verification Insight	5

What to Observe in GTKWave	5

Key Observations	5

ROM Content at Boot	6

8. Common Issues & Debug Tips	6

Debug Tip: Check Instruction Encoding	6

9. Summary	6

Fundamentals of RISC-V ISA

Course: PicoRV32 SoC with AXI-Lite & UART Integration

## 1. Title

Fundamentals of RISC-V ISA — Understanding Instruction Execution with PicoRV32

## 2. Objective

By the end of this module, learners will be able to:

- Explain the RISC-V base integer ISA (RV32I) and its register file

- Describe how instructions are encoded in 32-bit words

- Observe PicoRV32 CPU executing instructions through simulation

- Understand the native memory interface (mem_valid/mem_ready) used by PicoRV32

- Read and interpret basic RISC-V waveforms in GTKWave

## 3. Theory (Multi-Level)

### 3.1 Beginner — Simple Intuition

Think of RISC-V as a set of very simple cooking instructions your CPU follows, one at a time:

- ADD x1, x2, x3 → “Take the number in shelf x2, add the number in shelf x3, put the result on shelf x1”

- LW x4, 0(x1) → “Go to the address in x1, pick up a 32-bit value, put it in x4”

The CPU (PicoRV32) has 32 shelves (registers, named x0 – x31) and runs through instructions stored in memory one by one. It is a RISC (Reduced Instruction Set Computer) processor — meaning each instruction is small, simple, and takes a predictable amount of time.

### 3.2 Intermediate — Engineering View

RV32I Base ISA defines: - 32 general-purpose registers (x0–x31), each 32 bits wide. x0 is hardwired to zero. - 6 core instruction formats: R, I, S, B, U, J — all 32 bits wide - Instructions: load/store, arithmetic/logical, branches, jumps, and system calls

| Instruction Format | Fields | Example Instructions |
| --- | --- | --- |
| R-type | opcode, rd, funct3, rs1, rs2, funct7 | ADD, SUB, AND |
| I-type | opcode, rd, funct3, rs1, imm[11:0] | ADDI, LW, JALR |
| S-type | opcode, imm[4:0], funct3, rs1, rs2, imm[11:5] | SW, SB |
| B-type | opcode, offset, funct3, rs1, rs2 | BEQ, BNE |
| U-type | opcode, rd, imm[31:12] | LUI, AUIPC |
| J-type | opcode, rd, imm[20:1] | JAL |

Program Counter (PC): Starts at 0x00000000 (PROGADDR_RESET in PicoRV32). Every instruction fetch increments PC by 4 bytes.

### 3.3 Advanced — Signal/Architecture Level

PicoRV32 uses a native memory interface (not AXI at this stage):

Signal           Direction    Width   Description
-------          ---------    -----   -----------
mem_valid        CPU→Mem       1       CPU asserts when address is valid
mem_instr        CPU→Mem       1       High for instruction fetch
mem_ready        Mem→CPU       1       Memory asserts when data is ready
mem_addr         CPU→Mem      32       Address being accessed
mem_wdata        CPU→Mem      32       Write data
mem_wstrb        CPU→Mem       4       Byte write strobe (0xF=full word)
mem_rdata        Mem→CPU      32       Read data returned to CPU

The handshake is simple: CPU asserts mem_valid, keeps it high; memory processes the request and asserts mem_ready for one cycle; CPU latches mem_rdata. This is a stall-based protocol — if memory is slow, mem_ready stays low and the CPU pipeline stalls.

In the standalone PicoRV32 test (rtl/PicoRV32/), a simple on-chip memory answers immediately, making mem_ready follow mem_valid in one cycle.

## 4. Architecture / Concept

### PicoRV32 CPU Block Diagram

Key design parameters (from rtl/top.v instantiation):

.ENABLE_COUNTERS    (1),    // Cycle/instret counters enabled
.ENABLE_REGS_16_31  (1),    // Full 32-register file
.CATCH_MISALIGN     (1),    // Trap misaligned accesses
.CATCH_ILLINSN      (1),    // Trap illegal instructions
.PROGADDR_RESET     (32'h0000_0000),  // Boot address
.STACKADDR          (32'h0001_00FC)   // Stack top

### Instruction Flow (Fetch → Decode → Execute → Memory → Writeback)

Cycle 1:  mem_valid=1, mem_instr=1, mem_addr=0x0000_0000  (Instruction Fetch)
          ← mem_ready=1, mem_rdata=0x00020137             (LUI x2, 0x20)

Cycle 2:  Decode 0x00020137 → LUI x2, imm=0x20000
          Execute: x2 ← 0x0002_0000

Cycle 3:  mem_valid=1, mem_instr=1, mem_addr=0x0000_0004  (Next IF)
          ← mem_ready=1, mem_rdata=0xFFC10113             (ADDI x2, x2, -4)

## 5. Implementation

### Files in This Module

| File | Role |
| --- | --- |
| rtl/PicoRV32/picorv32.v | Main PicoRV32 CPU RTL (the processor itself) |
| rtl/PicoRV32/Memory.v | Simple instruction + data memory |
| rtl/PicoRV32/top.v | Top wrapper connecting CPU to memory |
| rtl/PicoRV32/tb_processor.v | Testbench for isolated PicoRV32 simulation |

### How It Works in This Project

- The standalone test (make pico) runs PicoRV32 with a dedicated instruction memory containing a simple RISC-V assembly program (arithmetic, loads, stores).

- The testbench (tb_processor.v) monitors mem_valid/mem_ready and dumps signals to tb_picorv32.vcd.

- The firmware used in Sub-modules 7/8 is compiled by riscv32-unknown-elf-gcc targeting RV32I and loaded into rom.hex.

## 6. Lab / Hands-on Section

### Prerequisites

# Ensure tools are installed
verilator --version   # >= 4.220
gtkwave --version
riscv32-unknown-elf-gcc --version

### Step-by-Step Commands

Step 1: Navigate to project root

cd /home/deep/nielit/projects/tested_projects/picorv32_soc

Step 2: Run the PicoRV32 standalone simulation

make pico

This does: 1. Builds the standalone RISC-V firmware in rtl/PicoRV32/ 2. Compiles all RTL + testbench with Verilator (--binary --trace --timing) 3. Runs the simulation binary → produces tb_picorv32.vcd 4. Opens GTKWave automatically

Step 3: Observe the simulation output

Expected terminal output:

[ROM] loaded from parameter: ...
-- Simulation Complete --

Step 4: View waveforms

make wave

or directly:

gtkwave tb_picorv32.vcd &

Step 5: Clean build artifacts

make clean

## 7. Waveform / Verification Insight

### What to Observe in GTKWave

Open tb_picorv32.vcd and add these signals to the waveform window:

clk
resetn
mem_valid
mem_instr
mem_ready
mem_addr [31:0]
mem_wdata [31:0]
mem_wstrb [3:0]
mem_rdata [31:0]

### Key Observations

| Signal Pattern | What It Means |
| --- | --- |
| mem_valid=1, mem_instr=1 | CPU is fetching an instruction |
| mem_valid=1, mem_instr=0, mem_wstrb≠0 | CPU is performing a store |
| mem_valid=1, mem_instr=0, mem_wstrb=0 | CPU is performing a load |
| mem_ready=0 for multiple cycles | Memory stall (pipeline waits) |
| mem_addr incrementing by 4 | Sequential instruction execution |

### ROM Content at Boot

The first instruction at 0x00000000 is 0x00020137 = LUI x2, 0x20, which sets up the stack pointer to 0x00020000. You can verify this in the waveform by checking mem_rdata when mem_addr=0x00000000.

## 8. Common Issues & Debug Tips

| Issue | Likely Cause | Fix |
| --- | --- | --- |
| make pico fails — tool not found | Verilator/GCC not in PATH | Source your tools: source ~/oss/environment |
| tb_picorv32.vcd not generated | Verilator --trace missing | Check VLFLAGS in Makefile includes --trace |
| Simulation ends immediately | ROM is all NOPs (0x00000013) | Check the firmware was compiled in rtl/PicoRV32/ |
| GTKWave shows flat signals | Wrong VCD file opened | Confirm file path; use ls -la *.vcd |
| CPU stalls forever | mem_ready never asserts | Check memory module wiring in top.v |
| trap signal goes high | Illegal instruction or misaligned access | Check firmware ELF for undefined behavior |

### Debug Tip: Check Instruction Encoding

To decode any mem_rdata value manually:

# Quick decoder (Python)
instr = 0x00020137
opcode = instr & 0x7F       # = 0x37 = LUI
rd     = (instr >> 7) & 0x1F # = x2
imm    = instr >> 12         # = 0x20
print(f"LUI x{rd}, 0x{imm:05X}")

## 9. Summary

| Topic | Key Takeaway |
| --- | --- |
| RISC-V ISA | 32-bit fixed-length instructions, 32 registers, 6 formats |
| PicoRV32 | Open-source RV32I implementation, simple stall-based memory interface |
| Native memory bus | mem_valid/mem_ready handshake; stall if memory is slow |
| Boot address | PROGADDR_RESET = 0x00000000; stack at 0x0001_00FC |
| Lab result | tb_picorv32.vcd shows full instruction pipeline including IF, EX, MEM |

This module establishes the foundation that all subsequent modules build upon. The PicoRV32 CPU’s native memory interface (mem_valid/mem_ready) is converted to AXI-Lite in Sub-module 5 — this translation is the core bridge of the entire SoC.

Next Sub-module → Sub-module 2: Introduction to AXI-Lite