A Brief Description of UART	2

1. Title	2

2. Objective	2

3. Theory (Multi-Level)	2

3.1 Beginner — Simple Intuition	2

3.2 Intermediate — Engineering View	2

3.3 Advanced — Signal/Architecture Level	3

TX Module (rtl/uart_tx.v) — Full FSM	3

RX Module (rtl/uart_rx.v) — Full FSM	3

4. Architecture / Concept	4

UART Module Hierarchy	4

Loopback Test Configuration	4

Baud Rate Timing Diagram (115200 baud @ 50 MHz)	4

5. Implementation	5

Files in This Module	5

UART Parameters (from RTL)	5

Key Design Decisions	5

6. Lab / Hands-on Section	5

Step 1: Run UART Standalone Simulation	5

Expected Terminal Output	5

Step 2: View UART Waveform	6

Step 3: Measure Bit Period	6

7. Waveform / Verification Insight	6

TX Side Observations	6

RX Side Observations	6

Loopback Timing Delay	7

8. Common Issues & Debug Tips	7

Debug Snippet: Manually Decode a UART Byte	7

9. Summary	8

A Brief Description of UART

Course: PicoRV32 SoC with AXI-Lite & UART Integration
Lab Commands: make uart → make wave

## 1. Title

UART Protocol — Serial Communication, 8N1 Frame Format, and Baud Rate

## 2. Objective

By the end of this module, learners will be able to:

- Explain the UART (Universal Asynchronous Receiver Transmitter) protocol

- Describe the 8N1 frame format (1 start bit, 8 data bits, 1 stop bit)

- Calculate baud rate and clock cycles per bit at any frequency

- Trace UART TX and RX FSM states in Verilog RTL

- Run standalone UART simulation and verify loopback functionality

## 3. Theory (Multi-Level)

### 3.1 Beginner — Simple Intuition

UART is the simplest way for two chips to talk to each other using just two wires: - TX (Transmit): Send data - RX (Receive): Receive data

Unlike most digital signals which are synchronized with a clock, UART uses agreed-upon timing (the baud rate) — like two people speaking at an agreed pace without a metronome.

A “byte” over UART looks like this on the wire:

Idle  Start  D0  D1  D2  D3  D4  D5  D6  D7  Stop  Idle
 1      0    ?   ?   ?   ?   ?   ?   ?   ?    1      1

- Idle: Wire sits at 1 (HIGH)

- Start bit: Always 0 — alerts the receiver that a byte is coming

- 8 data bits: Sent LSB first (bit 0 first, bit 7 last)

- Stop bit: Always 1 — signals end of frame

### 3.2 Intermediate — Engineering View

Baud Rate = number of bits per second. Common values: 9600, 115200 baud.

At 115200 baud with a 50 MHz clock:

CLKS_PER_BIT = 50,000,000 / 115,200 = 434 clock cycles per bit

This constant 434 appears directly in the RTL:

// From rtl/uart_tx.v
localparam integer CLKS_PER_BIT = CLK_FREQ / BAUD_RATE;
// 50_000_000 / 115_200 = 434

UART TX FSM states:

IDLE → START → DATA (bits 0–7) → STOP → IDLE

UART RX challenge: The receiver has no shared clock with the sender. It must: 1. Detect the falling edge of the start bit 2. Wait half a bit period (217 cycles) to sample at the centre of the start bit 3. Then sample every 434 cycles to hit the centre of each data bit

This mid-bit sampling is critical — timing errors accumulate differently if you sample at the edges.

### 3.3 Advanced — Signal/Architecture Level

#### TX Module (rtl/uart_tx.v) — Full FSM

State    Action                       Condition to advance
IDLE     tx=1 (idle high)             valid=1 → latch data → goto START
START    tx=0 (start bit)             clk_cnt == 433 → goto DATA
DATA     tx=shift_reg[0], shift right clk_cnt == 433 and bit_cnt==7 → goto STOP
STOP     tx=1 (stop bit)              clk_cnt == 433 → goto IDLE

Signal definitions:

input  [7:0] data;   // Byte to transmit
input        valid;  // Pulse: data is ready to send
output       ready;  // High when FSM is IDLE (tx is ready)
output reg   tx;     // Serial output

#### RX Module (rtl/uart_rx.v) — Full FSM

State    Action                              Condition to advance
IDLE     rx_s=0 detected → start            falling edge detected → goto START
START    wait CLKS_HALF_BIT (217 cycles)    if rx_s still 0 → goto DATA (glitch filter)
DATA     sample rx_s each CLKS_PER_BIT      bit_cnt==7 → goto STOP
STOP     wait CLKS_PER_BIT                  if rx_s=1 → data_out=shift_reg, data_valid=1

2-FF Synchronizer (metastability protection):

always @(posedge clk or posedge reset) begin
    rx_sync0 <= rx;       // First flip-flop
    rx_sync1 <= rx_sync0; // Second flip-flop
end
wire rx_s = rx_sync1;     // Synchronized RX input

This prevents metastability when the asynchronous rx input crosses clock domains.

## 4. Architecture / Concept

### UART Module Hierarchy

### Loopback Test Configuration

In standalone UART testing:

uart_top.uart_tx_out ──→ uart_top.uart_rx_in   (internally looped)

The testbench tb/tb_uart_top.v sends a byte through TX, which loops back to RX, and verifies the received byte matches.

### Baud Rate Timing Diagram (115200 baud @ 50 MHz)

Byte: 'A' = 0x41 = 0b00100001

Wire: 1 0 1 0 0 0 0 1 0 0 1  1
      ↑ ↑ ↑ ↑ ↑ ↑ ↑ ↑ ↑ ↑ ↑  ↑
      I S D0D1D2D3D4D5D6D7 T  I
      d t                 o  d
      l a                 p  l
      e r                    e

Each bit lasts 434 clock cycles (8.68 µs at 50 MHz).

## 5. Implementation

### Files in This Module

| File | Role |
| --- | --- |
| rtl/uart_tx.v | UART Transmitter — 8N1, FSM-based, parameterized baud rate |
| rtl/uart_rx.v | UART Receiver — 8N1, mid-bit sampling, 2-FF synchronizer |
| rtl/uart_top.v | Top wrapper for standalone UART test (TX + RX connected) |
| tb/tb_uart_top.v | Testbench driving bytes through UART TX and verifying RX |

### UART Parameters (from RTL)

parameter CLK_FREQ  = 50_000_000;   // 50 MHz system clock
parameter BAUD_RATE = 115_200;      // 115200 baud
// Derived: CLKS_PER_BIT = 50_000_000 / 115_200 = 434

### Key Design Decisions

- ASIC-clean: All registers reset via posedge reset — no = value inline initializers

- 2-FF synchronizer: rx_sync0 → rx_sync1 prevents metastability on async RX input

- Mid-bit sampling: RX waits CLKS_HALF_BIT - 1 before sampling start bit, then samples every CLKS_PER_BIT — robust against ±2.5% timing variation

- Glitch filter: If RX goes high during START state (before half-bit timeout), FSM returns to IDLE (false start rejection)

## 6. Lab / Hands-on Section

### Step 1: Run UART Standalone Simulation

cd /home/deep/nielit/projects/tested_projects/picorv32_soc
make uart

This command: 1. Cleans obj_uart/ 2. Compiles: uart_tx.v + uart_rx.v + uart_top.v + tb_uart_top.v 3. Runs ./obj_uart/sim_uart → produces tb_uart.vcd 4. Opens GTKWave

### Expected Terminal Output

--------------------------------------------------
 UART Standalone Simulation
--------------------------------------------------
[UART TB] Sending byte: 0x48 ('H')
[UART TB] Received: 0x48 ('H') ← PASS
[UART TB] Sending byte: 0x65 ('e')
[UART TB] Received: 0x65 ('e') ← PASS
...
[UART TB] String "hello deepak" verified: ALL PASS

### Step 2: View UART Waveform

gtkwave tb_uart.vcd &
# or use pre-saved layout:
gtkwave tb_uart.vcd -a uart.gtkw &

Add these signals:

clk
reset
uart_top.tx
uart_top.rx
uart_tx.state
uart_tx.clk_cnt[9:0]
uart_tx.shift_reg[7:0]
uart_tx.bit_cnt[2:0]
uart_rx.state
uart_rx.data_out[7:0]
uart_rx.data_valid

### Step 3: Measure Bit Period

In GTKWave: 1. Click on the tx signal 2. Place a marker at the falling edge of the start bit 3. Place another marker 434 cycles later 4. Verify the bit transitions align with marker spacing (≈ 8.68 µs)

## 7. Waveform / Verification Insight

### TX Side Observations

| Signal | What To See |
| --- | --- |
| tx | Starts HIGH (idle), drops to 0 (start), then 8 data bits LSB-first, then 1 (stop) |
| uart_tx.state | Cycles: IDLE(0) → START(1) → DATA(2) → STOP(3) → IDLE(0) |
| uart_tx.clk_cnt | Counts 0→433 in each bit, then resets |
| uart_tx.bit_cnt | Counts 0→7 during DATA state (8 data bits) |
| uart_tx.shift_reg | Right-shifts each cycle — MSB shifted out, 0 shifted in |

### RX Side Observations

| Signal | What To See |
| --- | --- |
| uart_rx.state | IDLE → START → DATA → STOP → IDLE |
| uart_rx.clk_cnt | Half-bit wait in START; full-bit wait in DATA/STOP |
| uart_rx.shift_reg | Builds up the received byte bit by bit |
| uart_rx.data_valid | Single-cycle pulse when byte is complete |
| uart_rx.data_out | Should match the transmitted byte |

### Loopback Timing Delay

In waveforms, notice that uart_rx.data_valid pulses approximately:

delay = (1 start + 8 data + 1 stop) × 434 cycles = 10 × 434 = 4340 cycles
     ≈ 4340 × 20 ns = 86.8 µs after TX starts

## 8. Common Issues & Debug Tips

| Issue | Likely Cause | Fix |
| --- | --- | --- |
| data_valid never pulses | RX not connected to TX in loopback | Check uart_top.v for correct TX→RX wire |
| Received byte is wrong | Bit sampling at edge instead of centre | Verify CLKS_HALF_BIT is correctly CLKS_PER_BIT/2 |
| Framing error (stop=0) | Clock frequency or baud rate mismatch | Verify parameters: CLK_FREQ=50_000_000, BAUD_RATE=115_200 |
| First bit always corrupted | 2-FF synchronizer not reset properly | Check rx_sync0/rx_sync1 reset to 1'b1 (idle) |
| TX sends garbage | shift_reg loaded wrong | Verify valid pulse triggers shift_reg <= data in IDLE state |
| GTKWave shows flat TX | valid never raised in testbench | Check testbench sequence starts the send correctly |

### Debug Snippet: Manually Decode a UART Byte

For BAUD_RATE=115200, CLK_FREQ=50MHz:

CLKS_PER_BIT = 434 cycles
1 frame = 10 bits × 434 cycles = 4340 cycles = 86.8 µs

In GTKWave: 1. Find falling edge of tx (start bit) 2. Jump 217 cycles → confirm tx=0 (centre of start bit) 3. Jump 434 cycles × 8 → sample each data bit at mid-point 4. Build byte from D0(LSB) to D7(MSB)

## 9. Summary

| Concept | Detail |
| --- | --- |
| Protocol | Asynchronous serial, no shared clock |
| Frame format | 8N1: 1 start + 8 data (LSB first) + 1 stop |
| Baud rate | 115200 baud → 434 cycles/bit at 50 MHz |
| TX FSM | IDLE → START → DATA(×8) → STOP → IDLE |
| RX FSM | IDLE → START (half-bit wait) → DATA(×8) → STOP → IDLE |
| Clock safety | 2-FF synchronizer prevents metastability on async RX input |
| Glitch filter | RX rejects false start if RX goes high within half-bit time |
| Lab result | Byte loopback verified: TX data == RX data_out |

UART is the output terminal of this SoC. Every uart_puts() call in firmware eventually becomes serial bit transitions on the uart_tx pin — visible in simulation as the [TB] RX byte: log lines.

Previous: Sub-module 2 — AXI-Lite
Next: Sub-module 4 — Memory-Mapped RISC-V + UART