Memory-Mapped Interface (MMI) of RISC-V with UART	2

1. Title	2

2. Objective	2

3. Theory (Multi-Level)	2

3.1 Beginner — Simple Intuition	2

3.2 Intermediate — Engineering View	3

3.3 Advanced — Signal/Architecture Level	3

4. Architecture / Concept	4

Block Diagram: MMI Design (No AXI)	4

CPU ↔ UART Communication Flow (Firmware Perspective)	4

Status Register Polling (uart_getc)	5

5. Implementation	6

Files in This Module	6

Firmware Key Functions (fw/main.c)	6

6. Lab / Hands-on Section	6

Step 1: Build Firmware Separately (Optional)	6

Step 2: Run the Full MMI SoC Simulation	7

Expected Terminal Output	7

Step 3: View Waveforms	7

7. Waveform / Verification Insight	8

Key Signals to Monitor	8

Observations	8

8. Common Issues & Debug Tips	8

Debugging MMIO: Address Watch	9

9. Summary	9

# Memory-Mapped Interface (MMI) of RISC-V with UART

Course: PicoRV32 SoC with AXI-Lite & UART Integration
Lab Commands: make soc_pico_uart_mem → make fw → make wave

## 1. Title

Memory-Mapped Interface (MMI) — Connecting PicoRV32 to UART Without AXI

## 2. Objective

By the end of this module, learners will be able to:

- Explain Memory-Mapped I/O (MMIO) and why it is foundational to embedded systems

- Map UART TX, RX, and STATUS registers to specific memory addresses in software

- Understand how the PicoRV32 mem_valid/mem_ready bus translates to peripheral register access

- Run the picorv32_uart_mem sub-project and observe CPU ↔ UART communication

- Write simple C firmware that reads/writes UART through fixed memory addresses

## 3. Theory (Multi-Level)

### 3.1 Beginner — Simple Intuition

In a typical embedded system, instead of having separate wires for each peripheral command, we use the memory address space as a control panel. Each peripheral is assigned a range of addresses:

- Write to address 0x10000000 → Send a byte over UART

- Read from address 0x10000004 → Get the received UART byte

- Read from address 0x10000008 → Check if data is available

This is called Memory-Mapped I/O (MMIO). The CPU doesn’t need special hardware for each peripheral — it just uses the same load/store instructions it uses for RAM.

Analogy: Imagine a hotel room with a panel of buttons. Pressing button #1 calls room service, button #2 calls housekeeping, button #3 dims the lights. The CPU presses a “button” (issues a store to an address) and the hardware responds.

### 3.2 Intermediate — Engineering View

In the picorv32_uart_mem design: - PicoRV32 generates mem_addr + mem_valid + mem_wstrb for every access - An address decoder routes the request to either ROM, SRAM, or UART based on the address - The UART peripheral accepts mem_wdata directly without AXI protocol overhead

UART Register Map (from fw/main.c):

#define UART_TX      (*(volatile char*)0x10000000)  // Write → transmit byte
#define UART_RX      (*(volatile char*)0x10000004)  // Read  → received byte
#define UART_STATUS  (*(volatile int*)0x10000008)   // Read  → [1]=RX ready

volatile keyword: Tells the C compiler not to optimize away hardware register accesses. Without it, the compiler might cache the value in a CPU register and never actually perform the memory-mapped access.

### 3.3 Advanced — Signal/Architecture Level

Address Range Decoding (direct mem-mapped, no AXI):

CPU addr [31:0] → Combinational decode:
  [31:16] == 16'h0000 AND [15] == 0  → ROM   (0x0000_0000 – 0x0000_7FFF)
  [31:16] == 16'h0001               → SRAM  (0x0001_0000 – 0x0001_FFFF)
  [31:28] == 4'h1                   → UART  (0x1000_0000 – 0x1000_000F)

UART access via direct mem bus:

// mem_uart: Write side
if (mem_valid && (|mem_wstrb) && uart_sel) begin
    uart_tx_data  <= mem_wdata[7:0];    // grab byte from CPU write
    uart_tx_valid <= 1'b1;              // trigger UART TX
end

// mem_uart: Read side
if (mem_valid && (!|mem_wstrb) && uart_sel) begin
    if (mem_addr[3:0] == 4'h8)
        mem_rdata <= {30'b0, rx_valid, 1'b0};   // status register
    else
        mem_rdata <= {23'b0, rx_valid, rx_data}; // RX data register
end

System memory map (full picture):

| Address Range | Peripheral | Interface |
| --- | --- | --- |
| 0x0000_0000 – 0x0000_7FFF | ROM (program) | Direct native bus |
| 0x0001_0000 – 0x0001_FFFF | SRAM (data) | Direct native bus |
| 0x1000_0000 – 0x1000_000F | UART | Direct native bus |

## 4. Architecture / Concept

### Block Diagram: MMI Design (No AXI)

### CPU ↔ UART Communication Flow (Firmware Perspective)

firmware main.c:
1. uart_puts("Hello Deepak from Nielit!\n")
   │
   ├─ uart_putc('H')
   │    └─ UART_TX = 'H'
   │         └─ SW x05, 0(x11)   [store byte to 0x10000000]
   │              └─ PicoRV32: mem_valid=1, mem_wstrb=0x1, mem_addr=0x10000000
   │                           mem_wdata=0x48
   │                            → UART starts transmitting 'H'
   │
   ├─ uart_putc('e')  (next iteration)
   ...

### Status Register Polling (uart_getc)

while (!(UART_STATUS & 0x2));  // Poll bit 1 = RX ready
return UART_RX;                // Read received byte

LW  a0, 8(uart_base)    ; load status register
ANDI a0, a0, 2          ; check bit 1 (RX ready)
BEQ  a0, x0, loop       ; retry if not ready
LW   a1, 4(uart_base)   ; load received byte

## 5. Implementation

### Files in This Module

| File | Role |
| --- | --- |
| picorv32_uart_mem/rtl/top.v | MMI SoC top — CPU + ROM + SRAM + UART (direct bus) |
| picorv32_uart_mem/rtl/uart_mem.v | Direct-bus UART peripheral (no AXI) |
| picorv32_uart_mem/rtl/mem_rom.v | ROM with native memory interface |
| picorv32_uart_mem/rtl/mem_sram.v | SRAM with native memory interface |
| fw/main.c | Firmware: prints greeting 10× then echo loop |
| fw/crt0.S | C runtime startup: stack init + call main |
| fw/link.ld | Linker script: ROM from 0x0, STACK at 0x1_00FC |
| picorv32_uart_mem/tb/tb_mem_soc.v | Testbench: UART monitor + PING test |

### Firmware Key Functions (fw/main.c)

#define UART_TX      (*(volatile char*)0x10000000)
#define UART_RX      (*(volatile char*)0x10000004)
#define UART_STATUS  (*(volatile int*)0x10000008)

static void uart_putc(char c) { UART_TX = c; }

static void uart_puts(const char *s) {
    while (*s) uart_putc(*s++);
}

static char uart_getc(void) {
    while (!(UART_STATUS & 0x2));  // Wait for RX ready
    return UART_RX;
}

int main(void) {
    for (int i = 0; i < 10; i++)
        uart_puts("Hello Deepak from Nielit!\n");
    while (1) uart_putc(uart_getc());  // Echo loop
}

## 6. Lab / Hands-on Section

### Step 1: Build Firmware Separately (Optional)

cd /home/deep/nielit/projects/tested_projects/picorv32_soc/fw
make
# Produces: firmware.elf, firmware.bin, rom.hex

The firmware build chain:

main.c + crt0.S → (riscv32-unknown-elf-gcc) → firmware.elf
firmware.elf → (objcopy) → firmware.bin
firmware.bin → (bin2hex.py) → rom.hex  (for $readmemh in RTL)

### Step 2: Run the Full MMI SoC Simulation

cd /home/deep/nielit/projects/tested_projects/picorv32_soc
make soc_pico_uart_mem

This command: 1. Builds firmware in picorv32_uart_mem/fw/ 2. Compiles the MMI SoC RTL + testbench with Verilator 3. Runs the simulation → produces tb_mem_soc.vcd/.fst 4. Opens GTKWave automatically

### Expected Terminal Output

--------------------------------------------------
 UART SoC Simulation Started
--------------------------------------------------

[TB] UART Monitor Started
--------------------------------------------
[TB] RX byte: H
[TB] RX byte: e
[TB] RX byte: l
[TB] RX byte: l
[TB] RX byte: o
...
[TB] Greeting message count = 10

============================================
 10 GREETING MESSAGES RECEIVED
 UART OUTPUT VERIFIED
============================================

--------------------------------------------
[TB] FIFO flushed – sending command : PING
--------------------------------------------
[TB] RX byte: P
[TB] RX byte: I
[TB] RX byte: N
[TB] RX byte: G

============================================
 TEST PASSED
 Echo received : PING
============================================

### Step 3: View Waveforms

make wave    # Opens tb_top.fst (or tb_mem_soc.fst for this target)

## 7. Waveform / Verification Insight

### Key Signals to Monitor

clk
reset
cpu_mem_valid
cpu_mem_instr
cpu_mem_ready
cpu_mem_addr[31:0]
cpu_mem_wstrb[3:0]
cpu_mem_wdata[31:0]
uart_sel          ← goes high when addr = 0x10000000
uart_tx           ← serial bitstream output
uart_mem.tx_valid ← TX trigger from CPU
uart_mem.rx_data_valid ← RX byte received

### Observations

- Instruction fetches: cpu_mem_instr=1, addr incrementing through ROM space (0x0000_0004, 0x0000_0008, …)

- UART TX write: cpu_mem_valid=1, cpu_mem_wstrb=4'b0001, cpu_mem_addr=0x1000_0000, cpu_mem_wdata[7:0] = 0x48 ('H')

- UART byte serialized: After the write, uart_tx transitions from 1→0 (start bit) then through 8 data bits over 4340 clock cycles

- Status poll: CPU repeatedly reads 0x1000_0008 until bit 1 is set (PING phase)

- Echo path: CPU reads 0x1000_0004, then writes it back to 0x1000_0000

## 8. Common Issues & Debug Tips

| Issue | Likely Cause | Fix |
| --- | --- | --- |
| soc_pico_uart_mem fails to build | Firmware not compiled | Run cd fw && make first |
| 0 greeting messages received | ROM not loaded | Check +romhex= plusarg path in Makefile |
| UART TX flat (never transitions) | CPU never reaches uart_putc call | Check stack and PC trace — bad linker script? |
| Status poll hangs forever | uart_status bit mapping wrong | Verify UART_STATUS bit 1 = RX ready in RTL |
| Echo sends wrong character | UART_RX reads stale data | Ensure rx_buf_valid clears after read |
| Simulation times out | CPU in infinite wait loop | Add $display monitoring of cpu_mem_addr to track CPU state |

### Debugging MMIO: Address Watch

In the testbench, add this always block for immediate feedback:

always @(posedge clk) begin
    if (mem_valid && mem_addr == 32'h10000000 && |mem_wstrb)
        $display("[DBG] UART TX write: 0x%02h ('%c')", mem_wdata[7:0], mem_wdata[7:0]);
end

## 9. Summary

| Concept | Detail |
| --- | --- |
| MMI | Peripherals mapped to CPU address space; accessed via load/store |
| UART registers | TX: 0x1000_0000 |
| volatile in C | Prevents compiler from caching hardware register reads |
| Address decode | Combinational logic maps address ranges to peripheral select signals |
| Blocking RX | uart_getc() polls STATUS bit 1 until RX data is available |
| Lab result | CPU prints 10× “Hello Deepak from Nielit!” then echoes “PING” → TEST PASSED |
| vs Sub-module 5+ | This Sub-module uses a direct native bus; Sub-module 5+ adds full AXI-Lite protocol |

This Sub-module bridges the gap between raw RISC-V ISA knowledge (Sub-module 1) and the full AXI-based SoC (Sub-modules 5–8). Understanding MMIO is essential for all embedded firmware development.

Previous: Sub-module 3 — UART
Next: Sub-module 5 — RISC-V AXI Integration