Sub-module 5: RISC-V AXI Integration	2

1. Title	2

2. Objective	2

3. Theory (Multi-Level)	2

3.1 Beginner — Simple Intuition	2

3.2 Intermediate — Engineering View	2

3.3 Advanced — Signal/Architecture Level	3

4. Architecture / Concept	5

Full Bridge Architecture	5

Timing: CPU to AXI Write (Best Case)	5

5. Implementation	5

Files in This Module	5

Bridge Registers (from rtl/top.v)	6

Demo Testbench (tb_riscv_axi_lite.v) — What It Does	6

6. Lab / Hands-on Section	6

Step 1: Run the RISC-V AXI Integration Demo	6

Expected Terminal Output	6

Step 2: Trace the AXI Write Handshake in GTKWave	7

7. Waveform / Verification Insight	8

Write Transaction Checkpoint	8

Read Transaction Checkpoint	8

Bridge FSM in Real SoC (tb_top.v context)	8

8. Common Issues & Debug Tips	8

Protocol Violation: VALID Must Not Be Withdrawn	9

9. Summary	10

# Sub-module 5: RISC-V AXI Integration

Course: PicoRV32 SoC with AXI-Lite & UART Integration
Lab Commands: make riscv_axi → make wave

## 1. Title

RISC-V AXI Integration — Bridging PicoRV32 Native Bus to AXI-Lite Protocol

## 2. Objective

By the end of this module, learners will be able to:

- Explain why a protocol bridge is needed between PicoRV32 and AXI peripherals

- Describe the CPU-to-AXI bridge state machine implemented in rtl/top.v

- Trace an AXI-Lite write/read transaction generated by the PicoRV32 native bus

- Run the make riscv_axi demo testbench and observe formatted handshake logs

- Identify the FSM states: ST_IDLE → ST_WR_AW → ST_WR_B → ST_RD_AR → ST_RD_R

## 3. Theory (Multi-Level)

### 3.1 Beginner — Simple Intuition

PicoRV32 speaks its own simple language: “here is an address and data, tell me when you are done.” The AXI peripheral speaks a more formal protocol: five separate conversations (channels) with specific handshake rules.

The AXI bridge is a translator that stands between them:

PicoRV32          AXI Bridge              AXI Slave
(native bus)   (protocol converter)      (ROM/SRAM/UART)

mem_valid=1  ──►  translates to ──►   awvalid=1 (write addr)
                                 + wvalid=1  (write data)
                  waits for         ◄── bvalid=1 (response)
mem_ready=1  ◄──  signals done ──────────────────

The CPU never “sees” AXI at all — it just uses its simple mem_valid/mem_ready protocol, and the bridge handles all the AXI complexity underneath.

### 3.2 Intermediate — Engineering View

Why is the bridge needed?

PicoRV32’s native memory interface is a single channel with one valid/ready pair. AXI-Lite has five independent channels. A direct connection is not possible.

The bridge must: 1. Detect if the CPU wants a read (wstrb=0) or write (wstrb≠0) 2. Launch the appropriate AXI channel(s) 3. Wait for AXI completion across multiple clock cycles 4. Assert mem_ready when the AXI transaction finishes 5. Return mem_rdata for reads

FSM States (from rtl/top.v):

ST_IDLE   (3'd0): Wait for cpu_mem_valid
ST_WR_AW  (3'd1): Issue AW+W channels, wait for B response
ST_WR_B   (3'd2): Wait for B channel (if AW/W already done)
ST_RD_AR  (3'd3): Issue AR channel, wait for arready
ST_RD_R   (3'd4): Wait for R channel data

### 3.3 Advanced — Signal/Architecture Level

Bridge internal signal mapping (rtl/top.v, lines 102–224):

// CPU outputs (inputs to bridge)
wire cpu_mem_valid;      // CPU has a pending request
wire [31:0] cpu_mem_addr;
wire [31:0] cpu_mem_wdata;
wire [3:0]  cpu_mem_wstrb;

// Bridge outputs (to CPU)
reg  ready_r;            // → cpu_mem_ready  (one-cycle pulse)
reg  [31:0] rdata_r;     // → cpu_mem_rdata  (latched read data)

assign cpu_mem_ready = ready_r;
assign cpu_mem_rdata = rdata_r;

Write Transaction FSM Detail:

ST_IDLE:
  if (cpu_mem_valid && |cpu_mem_wstrb):
    m_axi_awaddr  ← cpu_mem_addr
    m_axi_awvalid ← 1
    m_axi_wdata   ← cpu_mem_wdata
    m_axi_wstrb_r ← cpu_mem_wstrb
    m_axi_wvalid  ← 1
    → ST_WR_AW

ST_WR_AW:
  if (m_axi_awready && m_axi_awvalid): m_axi_awvalid ← 0
  if (m_axi_wready  && m_axi_wvalid):  m_axi_wvalid  ← 0
  m_axi_bready ← 1
  if (m_axi_bvalid && m_axi_bready):   ready_r ← 1 → ST_IDLE
  elif (!m_axi_awvalid && !m_axi_wvalid): → ST_WR_B

ST_WR_B:
  if (m_axi_bvalid): ready_r ← 1 → ST_IDLE

Read Transaction FSM Detail:

ST_IDLE:
  if (cpu_mem_valid && !|cpu_mem_wstrb):
    m_axi_araddr  ← cpu_mem_addr
    m_axi_arvalid ← 1
    → ST_RD_AR

ST_RD_AR:
  if (m_axi_arready && m_axi_arvalid):
    m_axi_arvalid ← 0
    m_axi_rready  ← 1
    → ST_RD_R

ST_RD_R:
  if (m_axi_rvalid && m_axi_rready):
    rdata_r      ← m_axi_rdata
    m_axi_rready ← 0
    ready_r      ← 1
    → ST_IDLE

## 4. Architecture / Concept

### Full Bridge Architecture

### Timing: CPU to AXI Write (Best Case)

Cycle:    1         2         3         4         5
CPU:      valid=1   valid=1   valid=1   ready=0   mem_ready=1
Bridge:   ST_IDLE   ST_WR_AW  (↓AW↓W)  ST_WR_B   ST_IDLE
AXI:      awvalid=1 awvalid=1 awready=1 bvalid=1  bready=1
          wvalid=1  wvalid=1  wready=1

The bridge introduces 2–4 cycle latency per AXI transaction, which is acceptable for MMIO register access.

## 5. Implementation

### Files in This Module

| File | Role |
| --- | --- |
| rtl/top.v | Contains the CPU-to-AXI bridge FSM (lines 91–224) |
| tb/tb_riscv_axi_lite.v | Self-contained demo TB showing step-by-step AXI handshake |
| rtl/axi_lite_interconnect.v | AXI interconnect that bridge connects to |
| rtl/axi_decoder.v | Address decoder used by interconnect |

### Bridge Registers (from rtl/top.v)

// ASIC-clean: no inline initializers — all driven by reset
reg  [31:0] m_axi_awaddr;   reg m_axi_awvalid;
reg  [31:0] m_axi_wdata;    reg [3:0] m_axi_wstrb_r; reg m_axi_wvalid;
reg         m_axi_bready;
reg  [31:0] m_axi_araddr;   reg m_axi_arvalid;
reg         m_axi_rready;
reg [2:0]   b_state;
reg [31:0]  rdata_r;
reg         ready_r;

### Demo Testbench (tb_riscv_axi_lite.v) — What It Does

This testbench simulates a CPU issuing an AXI write then read at address 0x1000_0000 (UART base): 1. Master asserts m_awaddr=0x10000000, m_awvalid=1, m_wdata=0xA5, m_wvalid=1 2. Slave responds: s_awready=1, s_wready=1 → write data stored in slave_reg[0] 3. Slave asserts s_bvalid=1 (OKAY response) 4. Master reads back: m_arvalid=1 → s_arready=1 → s_rdata=0xA5 → s_rvalid=1 5. Checks captured_rdata == TEST_DATA → prints “Success”

## 6. Lab / Hands-on Section

### Step 1: Run the RISC-V AXI Integration Demo

cd /home/deep/nielit/projects/tested_projects/picorv32_soc
make riscv_axi

This: 1. Cleans obj_axi/ 2. Compiles only tb/tb_riscv_axi_lite.v (self-contained, no RTL files needed) 3. Runs ./obj_axi/sim_axi → produces tb_riscv_axi_lite.vcd 4. Opens GTKWave

### Expected Terminal Output

---- Simulation Start ----

[0-10 ns] System in reset
[10 ns] Reset released → CPU and AXI start working

-------------------------------
[50 ns] AXI WRITE TRANSACTION
-------------------------------
CPU wants to write data

Step 1: Send Address
  AWADDR  = 0x10000000
  AWVALID = 1  → CPU says "address is ready"
  AWREADY = 1  → Bus says "address accepted"

Step 2: Send Data
  WDATA   = 0x000000A5
  WVALID  = 1  → CPU says "data is ready"
  WREADY  = 1  → Bus says "data accepted"

Step 3: Write Response
  BVALID  = 1  → Bus says "write completed"
  BREADY  = 1  → CPU accepts response

 Result: Data 0xA5 successfully written to address 0x10000000

-------------------------------
[110 ns] AXI READ TRANSACTION
-------------------------------
CPU wants to read data

Step 1: Send Address
  ARADDR  = 0x10000000
  ARVALID = 1  → CPU requests read
  ARREADY = 1  → Bus accepts request

Step 2: Get Data
  RDATA   = 0x000000A5
  RVALID  = 1  → Bus sends data
  RREADY  = 1  → CPU accepts data

 Result: Read value = 0xA5 (same as written)

-------------------------------
Final Conclusion:
Write = Success
Read  = Success
AXI-Lite integration is working correctly

---- Simulation End ----

### Step 2: Trace the AXI Write Handshake in GTKWave

gtkwave tb_riscv_axi_lite.vcd &

Add:

clk
reset
m_awaddr[31:0]    ← master write address
m_awvalid
s_awready         ← slave accepts address
m_wdata[31:0]     ← master write data
m_wvalid
s_wready          ← slave accepts data
s_bvalid          ← slave response
m_bready          ← master ready for response
m_araddr[31:0]    ← master read address
m_arvalid
s_arready         ← slave accepts address
s_rdata[31:0]     ← slave read data
s_rvalid          ← slave data valid
m_rready          ← master ready for data

## 7. Waveform / Verification Insight

### Write Transaction Checkpoint

In GTKWave, look for the cycle where m_awvalid AND s_awready = 1 simultaneously — this is the AW handshake. Measure the delay from m_awvalid rising to s_awready rising. In this testbench it is 1 cycle (ideal slave).

### Read Transaction Checkpoint

Look for m_arvalid AND s_arready = 1 → AR handshake. Then s_rvalid AND m_rready = 1 → R handshake. Confirm s_rdata = 0xA5 at the R handshake moment.

### Bridge FSM in Real SoC (tb_top.v context)

When running make soc, add b_state[2:0] to see the bridge FSM:

State value   Meaning
0 (ST_IDLE)   Waiting for cpu_mem_valid
1 (ST_WR_AW)  AW+W presented to interconnect
2 (ST_WR_B)   Waiting for B (write response)
3 (ST_RD_AR)  AR presented to interconnect
4 (ST_RD_R)   Waiting for R (read data)

The transition 0→1→0 or 0→3→4→0 visible in waveforms corresponds directly to each CPU instruction that accesses memory.

## 8. Common Issues & Debug Tips

| Issue | Likely Cause | Fix |
| --- | --- | --- |
| Bridge stuck in ST_WR_AW | m_axi_awready never asserts | Check interconnect routing — is wr_sel correct? |
| Bridge stuck in ST_RD_R | m_axi_rvalid never asserts | Check slave’s read path — is r_valid_r never set? |
| mem_ready never pulses | Bridge FSM never reaches ready_r ← 1 | Trace b_state in waveforms |
| CPU sees wrong rdata | rdata_r captured at wrong cycle | Ensure rdata_r ← m_axi_rdata happens only when rvalid=1 AND rready=1 |
| Bridge issues both AW and AR | CPU wstrb ambiguous | Confirm decoder uses ¦cpu_mem_wstrb != 0 → write |
| GTKWave shows no transitions | Wrong VCD file | Use ls -la *.vcd to verify tb_riscv_axi_lite.vcd was generated |

### Protocol Violation: VALID Must Not Be Withdrawn

## 9. Summary

| Concept | Detail |
| --- | --- |
| Bridge purpose | Converts PicoRV32 native (mem_valid/ready) to AXI-Lite 5-channel protocol |
| FSM states | ST_IDLE, ST_WR_AW, ST_WR_B, ST_RD_AR, ST_RD_R |
| Write latency | 3–4 cycles (AW+W handshake + B response) |
| Read latency | 2–3 cycles (AR handshake + R data) |
| Bridge location | Embedded in rtl/top.v (lines 91–224) |
| ASIC-clean | All bridge registers reset-driven; no inline initializers |
| Lab file | tb/tb_riscv_axi_lite.v — fully self-contained, no RTL dependencies |
| Lab result | Write 0xA5 to 0x10000000, read back 0xA5 → Success |

This module forms the critical connection between the RISC-V ISA world (Sub-modules 1–4) and the AXI peripheral world (Sub-modules 6–8). Every firmware memory access in the final SoC passes through this bridge.

Previous: Sub-module 4 — MMI RISC-V + UART
Next: Sub-module 6 — AXI + UART Integration