AXI + UART Integration	2

1. Title	2

2. Objective	2

3. Theory (Multi-Level)	2

3.1 Beginner — Simple Intuition	2

3.2 Intermediate — Engineering View	2

3.3 Advanced — Signal/Architecture Level	3

4. Architecture / Concept	4

uart_axi Internal Block Diagram	4

AXI-UART Data Flow (Full End-to-End)	5

5. Implementation	5

Files in This Module	5

Key Code Paths in rtl/uart_axi.v	6

6. Lab / Hands-on Section	6

Step 1: Run AXI-UART Integration Testbench	6

Expected Terminal Output	6

Step 2: View AXI-UART Waveform	7

7. Waveform / Verification Insight	8

AXI Write → UART TX Timeline	8

Backpressure Observation	8

RX Path Timeline	8

8. Common Issues & Debug Tips	8

Debug: Trace buf_valid Signal	9

9. Summary	9

# AXI + UART Integration

Course: PicoRV32 SoC with AXI-Lite & UART Integration
Lab Commands: make axi_uart → make wave

## 1. Title

AXI + UART Integration — UART Peripheral as an AXI-Lite Slave

## 2. Objective

By the end of this module, learners will be able to:

- Explain how a UART peripheral is exposed as an AXI-Lite slave

- Map the uart_axi module’s internal registers to AXI addresses

- Trace a complete AXI write → UART TX byte-serialization path

- Trace a complete UART RX → AXI read path

- Run the make axi_uart testbench and observe data flow through the AXI-UART pipeline

## 3. Theory (Multi-Level)

### 3.1 Beginner — Simple Intuition

Imagine you want to use a UART peripheral in an AXI-based SoC. The UART speaks “serial bits”, the AXI bus speaks “parallel register reads/writes”. We need an adapter:

- You write to AXI address 0x10000000 → the adapter stores the byte and triggers UART TX

- UART receives a byte → the adapter stores it → you read AXI address 0x10000004 to get it

- You read AXI address 0x10000008 → the adapter tells you if new data is waiting

The uart_axi.v module is this adapter. Externally it looks like a simple register bank to the AXI bus; internally it drives the uart_tx.v and reads from uart_rx.v.

### 3.2 Intermediate — Engineering View

uart_axi.v Register Map:

| AXI Address | Access | bit[8] | bits[7:0] | Description |
| --- | --- | --- | --- | --- |
| 0x10000000 | Write | — | TX data | Write byte to transmit |
| 0x10000004 | Read | rx_valid | rx_data | Read received byte (clears rx_buf_valid) |
| 0x10000008 | Read | — | {rx_valid, 1'b0} | Status: bit[1]=rx_buf_valid |

Write path (AXI → UART TX):

AXI master write to 0x1000_0000
    → uart_axi captures (aw_got=1, w_got=1)
    → w_byte ← wdata[7:0] (based on wstrb)
    → buf_data ← w_byte; buf_valid ← 1
    → uart_tx gets: data=buf_data, valid=1
    → 8N1 serial bitstream on tx_out pin
    → B response sent back: OKAY (2'b00)

Read path (UART RX → AXI):

uart_rx detects start bit on uart_rx pin
    → uart_rx shifts in 8 bits
    → data_valid pulse → rx_buf_data ← received byte; rx_buf_valid ← 1
AXI master read from 0x1000_0004
    → uart_axi returns {23'b0, rx_buf_valid, rx_buf_data}
    → rx_buf_valid ← 0 (cleared on data register read)

### 3.3 Advanced — Signal/Architecture Level

Write backpressure mechanism (ASIC-clean, from rtl/uart_axi.v):

// Ready assertions — held LOW when buffer is full
assign s_axi_awready = !aw_got && !buf_valid;
assign s_axi_wready  = !w_got  && !buf_valid;

This ensures that if the 1-byte TX buffer is full (buf_valid=1), the AXI master is stalled at the write data channel — AWREADY and WREADY go low, forcing the CPU bridge to wait. This prevents silent byte drops.

TX buffer state machine:

Reset:    buf_valid=0, buf_data=0

If tx_ready && buf_valid:       → tx_data←buf_data, tx_valid←1, buf_valid←0
If aw_got && w_got && !b_vld && !buf_valid: → buf_data←w_byte, buf_valid←1

(These two conditions interleave: UART drains buffer    )
(as fast as possible, AXI refills when buffer is empty  )

WSTRB byte selection:

if      (s_axi_wstrb[0]) w_byte <= s_axi_wdata[ 7: 0];  // byte 0
else if (s_axi_wstrb[1]) w_byte <= s_axi_wdata[15: 8];  // byte 1
else if (s_axi_wstrb[2]) w_byte <= s_axi_wdata[23:16];  // byte 2
else                     w_byte <= s_axi_wdata[31:24];  // byte 3

This allows 8-bit firmware stores (SB instruction, wstrb=4’h1) to correctly target the lower byte of the 32-bit AXI data word.

Module parameters:

module uart_axi #(
    parameter CLK_FREQ  = 50_000_000,   // System clock
    parameter BAUD_RATE = 115_200       // Serial baud rate
)
// CLKS_PER_BIT = 50_000_000 / 115_200 = 434 cycles per bit

## 4. Architecture / Concept

### uart_axi Internal Block Diagram

### AXI-UART Data Flow (Full End-to-End)

## 5. Implementation

### Files in This Module

| File | Role |
| --- | --- |
| rtl/uart_axi.v | UART peripheral as AXI-Lite slave — TX buffer + RX buffer + AXI controller |
| rtl/uart_tx.v | Instantiated inside uart_axi as u_tx |
| rtl/uart_rx.v | Instantiated inside uart_axi as u_rx |
| tb/tb_axi_lite.v | Testbench: AXI master + uart_axi DUT + UART serial monitor |
| tb/tb_axi_lite_master.v | Reusable AXI master task module |

### Key Code Paths in rtl/uart_axi.v

Write channel ready conditions (lines 172–173):

assign s_axi_awready = !aw_got && !buf_valid;
assign s_axi_wready  = !w_got  && !buf_valid;

RX read data format (lines 237–241):

if (s_axi_araddr[3:0] == 4'h8)
    r_data <= {30'b0, rx_buf_valid, 1'b0};   // Status register
else
    r_data <= {23'b0, rx_buf_valid, rx_buf_data}; // Data register: [8]=valid, [7:0]=data

Port naming note: The TX output is named tx_out (not uart_tx) to avoid Yosys scope-resolution collision with the submodule type name uart_tx. This is a real ASIC synthesis concern documented in the code:

output tx_out,  // renamed: avoids collision with submodule type `uart_tx`

## 6. Lab / Hands-on Section

### Step 1: Run AXI-UART Integration Testbench

cd /home/deep/nielit/projects/tested_projects/picorv32_soc
make axi_uart

This: 1. Cleans obj_axilite/ 2. Compiles: uart_tx.v + uart_rx.v + uart_axi.v + tb_axi_lite_master.v + tb_axi_lite.v 3. Runs ./obj_axilite/sim_axilite → produces tb_axi_lite.vcd 4. Opens GTKWave

### Expected Terminal Output

--------------------------------------------------
 AXI-UART Testbench Started
--------------------------------------------------
[AXI WRITE] addr=0x10000000 data=0x48 ('H') → Writing to UART TX
[AXI WRITE] awvalid=1 → Sent address
[AXI WRITE] awready=1 → Address accepted
[AXI WRITE] wvalid=1  → Sent data 0x00000048
[AXI WRITE] wready=1  → Data accepted
[AXI WRITE] bvalid=1  → Response: OKAY

[UART TX] Transmitting: 0x48 ('H')
[UART TX] Start bit → 8 data bits → Stop bit

[AXI READ] addr=0x10000008 → Reading UART status
[AXI READ] rdata=0x00000002 → RX data available

[AXI READ] addr=0x10000004 → Reading UART RX data
[AXI READ] rdata=0x00000148 → rx_valid=1, rx_data=0x48 ('H')
[TEST] Loopback data matches → PASS

### Step 2: View AXI-UART Waveform

gtkwave tb_axi_lite.vcd &

Add these signals in GTKWave:

clk                     ← System clock
reset

# AXI Write channel
s_axi_awaddr[31:0]
s_axi_awvalid
s_axi_awready           ← Stalls when buf_valid=1
s_axi_wdata[31:0]
s_axi_wvalid
s_axi_wready
s_axi_bvalid
s_axi_bready

# AXI Read channel
s_axi_araddr[31:0]
s_axi_arvalid
s_axi_arready
s_axi_rdata[31:0]
s_axi_rvalid
s_axi_rready

# UART TX internals
u_tx.tx                 ← Serial output (observe 8N1 frame)
u_tx.state              ← IDLE/START/DATA/STOP
u_tx.clk_cnt[9:0]       ← Baud rate counter (0–433)

# UART buffer
uart_axi.buf_valid      ← TX buffer occupancy
uart_axi.aw_got         ← AW channel captured
uart_axi.w_got          ← W channel captured
uart_axi.rx_buf_valid   ← RX data waiting

## 7. Waveform / Verification Insight

### AXI Write → UART TX Timeline

In GTKWave, look for this sequence: 1. s_axi_awvalid=1 (master presents write address) 2. s_axi_awready=1 (AW handshake) — uart_axi.aw_got pulses high 3. s_axi_wvalid=1 → s_axi_wready=1 (W handshake) — uart_axi.w_got pulses high 4. s_axi_bvalid=1 (B response OKAY) 5. uart_axi.buf_valid=1 → triggers uart_tx.valid=1 6. u_tx.tx drops to 0 (start bit) → 8 data bits → returns to 1 (stop bit)

### Backpressure Observation

Send two bytes back-to-back and observe: - First byte: awready=1, wready=1 immediately - Second byte (during first TX): awready=0, wready=0 while buf_valid=1 - Once uart_tx.ready=1 (UART idle), buf_valid=0 → awready and wready return to 1

This backpressure mechanism prevents data loss.

### RX Path Timeline

- uart_rx detects falling edge on uart_rx pin

- After 4340 cycles, data_valid=1 pulses

- uart_axi.rx_buf_valid=1, uart_axi.rx_buf_data = received_byte

- AXI read of 0x10000004 → rdata = {23'b0, 1'b1, received_byte}

- rx_buf_valid clears after the read handshake

## 8. Common Issues & Debug Tips

| Issue | Likely Cause | Fix |
| --- | --- | --- |
| UART TX never transmits | buf_valid never goes high | Confirm AW+W handshake completed before checking |
| UART TX sends wrong byte | w_byte used wrong wstrb lane | Check which WSTRB bit is 1 in your test vector |
| AXI write stalls permanently | buf_valid stuck high | Check uart_tx.ready — is UART TX FSM completing? |
| awready stays low forever | aw_got is stuck 1 | Did B-channel response complete? Check b_vld |
| RX data is 0x00 always | RX loopback not connected in TB | Check uart_axi.uart_rx is connected to TX in loopback TB |
| Status register reads 0 | RX byte not yet received | Add delay before status read (UART byte = 86.8 µs at 115200) |
| tx_out port name issue | Yosys collision with uart_tx module type | Already fixed: port renamed tx_out in rtl/uart_axi.v |

### Debug: Trace buf_valid Signal

// Add in testbench for live debug:
always @(posedge clk) begin
    if (dut.buf_valid && dut.tx_ready)
        $display("[DBG] TX byte sent: 0x%02h", dut.buf_data);
    if (dut.rx_buf_valid)
        $display("[DBG] RX byte ready: 0x%02h", dut.rx_buf_data);
end

## 9. Summary

| Concept | Detail |
| --- | --- |
| uart_axi role | Wraps uart_tx+uart_rx with AXI-Lite slave interface |
| TX write addr | 0x10000000 → byte from wdata[7:0] → 1-byte buffer → uart_tx |
| RX data addr | 0x10000004 → returns {rx_valid[8], rx_data[7:0]} |
| Status addr | 0x10000008 → returns {rx_valid[1], 1'b0} |
| Backpressure | awready=wready=0 when buf_valid=1 — zero data loss |
| Port naming | tx_out (not uart_tx) to avoid Yosys scope collision |
| ASIC-clean | No inline reg initializers; all state reset-driven |
| Lab result | AXI write → UART byte serialized; RX loopback → AXI read verified |

This module provides the complete “last mile” between the AXI bus and the physical UART serial lines. Combined with Sub-module 5 (CPU-AXI bridge) and Sub-module 7 (firmware), this forms the complete software-to-wire data path of the SoC.

Previous: Sub-module 5 — RISC-V AXI Integration
Next: Sub-module 7 — Firmware Implementation