Firmware Implementation	2

1. Title	2

2. Objective	2

3. Theory (Multi-Level)	2

3.1 Beginner — Simple Intuition	2

3.2 Intermediate — Engineering View	2

3.3 Advanced — Signal/Architecture Level	3

4. Architecture / Concept	4

Firmware Build Flow	4

Application Logic (fw/main.c)	5

Register Access in Assembly (Generated by Compiler)	5

5. Implementation	5

Files in This Module	5

Firmware Size Analysis	6

6. Lab / Hands-on Section	6

Step 1: Build Firmware	6

Step 2: Inspect the Compiled Firmware	7

Step 3: View ROM Hex Content	7

Step 4: Run SoC with this Firmware	7

Step 5: Verify Firmware is Loaded	7

7. Waveform / Verification Insight	7

Instruction Fetch Trace	7

UART TX Activity	8

Firmware String in ROM	8

8. Common Issues & Debug Tips	8

Verify RISC-V ABI with objdump	8

9. Summary	9

# Firmware Implementation

Course: PicoRV32 SoC with AXI-Lite & UART Integration
Lab Commands: make fw → make soc_pico_uart_mem

## 1. Title

Firmware Implementation — Writing Embedded C for PicoRV32, UART, and AXI-Lite SoC

## 2. Objective

By the end of this module, learners will be able to:

- Write bare-metal embedded C firmware for a RISC-V SoC with no OS

- Understand startup code (crt0.S) and linker scripts (link.ld)

- Implement UART driver functions using memory-mapped register access

- Compile firmware using riscv32-unknown-elf-gcc toolchain

- Convert firmware binary to ROM hex format for simulation and synthesis

## 3. Theory (Multi-Level)

### 3.1 Beginner — Simple Intuition

When you write C code for a PC, the operating system provides everything: printf(), file access, memory management. When writing firmware for a bare-metal RISC-V SoC, there is no OS. You must:

- Set up the stack yourself (where does the CPU put local variables?)

- Write directly to hardware registers using fixed addresses

- Build a tiny “stub” (crt0.S) that runs before main()

Think of it like writing a recipe where you also have to build the kitchen from scratch.

### 3.2 Intermediate — Engineering View

Bare-metal firmware components:

| Component | File | Purpose |
| --- | --- | --- |
| C runtime startup | fw/crt0.S | Initialize stack, call main(), trap on exit |
| Application code | fw/main.c | Application logic (UART echo) |
| Linker script | fw/link.ld | Map code/data to ROM/SRAM addresses |
| Firmware Makefile | fw/Makefile | Compile → link → objcopy → bin2hex |
| Hex converter | fw/bin2hex.py | Convert binary to 4-byte-wide hex for $readmemh |

Memory layout (from fw/link.ld):

MEMORY {
    ROM  (rx)  : ORIGIN = 0x00000000, LENGTH = 32K
    SRAM (rwx) : ORIGIN = 0x00010000, LENGTH = 8K
}

SECTIONS {
    .text  → ROM   (0x0000_0000): code + read-only data
    .data  → SRAM  (0x0001_0000): initialized global variables
    .bss   → SRAM               : zero-initialized globals
    Stack                        : top at 0x0001_00FC (descends)
}

UART register macros (fw/main.c):

#define UART_TX     (*(volatile char*)0x10000000)
#define UART_RX     (*(volatile char*)0x10000004)
#define UART_STATUS (*(volatile int*) 0x10000008)

volatile is mandatory — it prevents the compiler from caching the register read in a CPU register and bypassing the actual memory-mapped access.

### 3.3 Advanced — Signal/Architecture Level

Startup Sequence (crt0.S → main()):

# crt0.S: executed before main()
_start:
    # 1. Initialize stack pointer
    lui  sp, %hi(__stack_top)
    addi sp, sp, %lo(__stack_top)
    # __stack_top = 0x0001_00FC (from link.ld)

    # 2. Zero-initialize BSS section
    la   a0, __bss_start
    la   a1, __bss_end
bss_loop:
    bgeu a0, a1, bss_done
    sw   x0, 0(a0)
    addi a0, a0, 4
    j    bss_loop
bss_done:

    # 3. Call main
    call main

    # 4. Trap if main returns (should not happen)
_exit:
    j _exit

Compilation flags:

CFLAGS = -march=rv32i -mabi=ilp32 -nostartfiles -nostdlib
         -T link.ld -Wall -O2

- -march=rv32i: Target RV32I base ISA (no multiply/float — matches ENABLE_MUL=0 in PicoRV32)

- -mabi=ilp32: 32-bit integer, 32-bit pointers ABI

- -nostartfiles: Don’t link GCC’s crt0 (we provide our own)

- -nostdlib: Don’t link libc (we implement everything ourselves)

ROM hex format (rom.hex): Each line is one 32-bit little-endian instruction word in hex:

00020137  ← LUI  x2, 0x20  (first instruction: setup stack pointer upper)
FFC10113  ← ADDI x2, x2, -4
...

## 4. Architecture / Concept

### Firmware Build Flow

### Application Logic (fw/main.c)

main()
 │
 ├─ for i = 0..9:
 │    uart_puts("Hello Deepak from Nielit!\n")
 │        └─ uart_putc(each char)
 │               └─ UART_TX = char  [→ AXI write to 0x10000000]
 │
 └─ while(1):               ← Echo loop
       c = uart_getc()
       │   └─ while(!(UART_STATUS & 0x2));  [poll 0x10000008]
       │      return UART_RX               [read 0x10000004]
       │
       uart_putc(c)
           └─ UART_TX = c   [→ AXI write to 0x10000000]

### Register Access in Assembly (Generated by Compiler)

For UART_TX = 'H' (C store through volatile pointer):

lui   a0, 0x10000      # a0 = 0x10000000
li    a1, 'H'          # a1 = 0x48
sb    a1, 0(a0)        # store byte → mem[0x10000000] = 0x48
                       # mem_wstrb = 0x1 (byte store)

For uart_getc() status poll:

lui   a0, 0x10000      # a0 = 0x10000000
lw    t0, 8(a0)        # t0 = mem[0x10000008] = STATUS
andi  t0, t0, 2        # check bit 1
beq   t0, x0, -8      # if not ready, loop
lb    a0, 4(a0)        # return mem[0x10000004] = RX byte

## 5. Implementation

### Files in This Module

| File | Purpose |
| --- | --- |
| fw/main.c | Application: greeting + echo loop (201 lines, well documented) |
| fw/crt0.S | Startup: stack init + call main + trap |
| fw/link.ld | Linker script: ROM at 0x0, SRAM at 0x10000, stack at 0x1_00FC |
| fw/Makefile | Build recipe: compile → link → binary → hex |
| fw/bin2hex.py | Converts firmware binary to 4-byte hex words for $readmemh |
| fw/gen_rom_case.py | Generates synthesizable ROM case-statement from rom.hex |
| fw/firmware.elf | Pre-built ELF (for inspection with objdump) |
| fw/firmware.bin | Raw binary (167 bytes = ~42 instructions) |

### Firmware Size Analysis

riscv32-unknown-elf-size fw/firmware.elf

Expected output:

text    data     bss     dec     hex filename
    132      35       0     167      a7 firmware.elf

- .text = 132 bytes ≈ 33 instructions

- .data = 35 bytes = “Hello Deepak from Nielit!” string (26 chars + terminator × alignment)

## 6. Lab / Hands-on Section

### Step 1: Build Firmware

cd /home/deep/nielit/projects/tested_projects/picorv32_soc
make fw

This runs fw/Makefile:

# 1. Compile + link
riscv32-unknown-elf-gcc \
    -march=rv32i -mabi=ilp32 \
    -nostartfiles -nostdlib \
    -T link.ld \
    -o firmware.elf crt0.S main.c

# 2. Create binary
riscv32-unknown-elf-objcopy -O binary firmware.elf firmware.bin

# 3. Generate hex for simulation
python3 bin2hex.py firmware.bin > ../rtl/rom.hex

# 4. Generate synthesizable ROM case statement
python3 gen_rom_case.py rom.hex > rom_case.v

### Step 2: Inspect the Compiled Firmware

# Disassemble the ELF
riscv32-unknown-elf-objdump -d fw/firmware.elf

Expected output (first lines):

Disassembly of section .text:

00000000 <_start>:
   0: 00020137   lui  sp,0x20       # sp = 0x00020000
   4: ffc10113   addi sp,sp,-4     # sp = 0x0001FFFC (stack top)
   8: 00010297   auipc t0,0x10
   ...

### Step 3: View ROM Hex Content

cat rtl/rom.hex

00020137
FFC10113
00010297
FF828293
...
6C6C6548    ← ASCII 'lleH' (little-endian "Hell")
6544206F    ← ASCII 'eD o' 
6B617065    ← ASCII 'kape'
...

### Step 4: Run SoC with this Firmware

make soc_pico_uart_mem    # Uses picorv32_uart_mem sub-project
# OR
make soc                   # Uses AXI-based SoC design

### Step 5: Verify Firmware is Loaded

Add to testbench (or check existing logs):

[ROM] loaded from plusarg: .../rtl/rom.hex
[ROM] Loaded 42 words

## 7. Waveform / Verification Insight

### Instruction Fetch Trace

In the SoC waveform (make wave), observe: 1. After reset, cpu_mem_addr = 0x0000_0000 (fetching instruction 0 = LUI sp, 0x20) 2. cpu_mem_addr = 0x0000_0004 (fetching instruction 1 = ADDI sp, sp, -4) 3. Sequential fetches until the loop at uart_puts call site

### UART TX Activity

Look for clusters of cpu_mem_addr = 0x1000_0000 writes with mem_wstrb = 0x1: - First cluster: 26 writes for “Hello Deepak from Nielit!” (×10 = 260 total) - Then: reads at 0x1000_0008 (status polling) - Then: 4 writes at 0x1000_0000 (echo of PING characters)

### Firmware String in ROM

In GTKWave when reading ROM during .data load (AUIPC + copy loop): - cpu_mem_rdata will show 0x6C6C6548 = little-endian “Hell” segment of the string

## 8. Common Issues & Debug Tips

| Issue | Likely Cause | Fix |
| --- | --- | --- |
| make fw fails: compiler not found | GCC toolchain not in PATH | export PATH=$PATH:/path/to/riscv-gcc/bin |
| firmware.elf created but rom.hex wrong | bin2hex.py encoding error | Check Python3 is used; run python3 fw/bin2hex.py fw/firmware.bin manually |
| CPU traps immediately | Stack pointer initialization failed | Check link.ld STACK_TOP address matches PicoRV32 STACKADDR parameter |
| SB (byte store) writes 32 bits | Compiler generated word store | Add volatile char* cast explicitly: UART_TX = c where UART_TX is volatile char* |
| String appears garbled | ROM hex not updated | Run make fw again after modifying firmware |
| CPU loops forever before printing | crt0.S BSS clear corrupts UART reg | Ensure BSS region does not overlap UART address (it doesn’t: SRAM is at 0x10000) |
| Missing -march=rv32i | Division instruction generated | PicoRV32 has ENABLE_DIV=0 — rv32i forbids compiler from emitting divide |

### Verify RISC-V ABI with objdump

riscv32-unknown-elf-objdump -h fw/firmware.elf

Check: - .text VMA = 0x00000000 (in ROM) - .data VMA = 0x00010000 (in SRAM) — but LMA = somewhere in ROM (copied by startup)

## 9. Summary

| Topic | Detail |
| --- | --- |
| Toolchain | riscv32-unknown-elf-gcc with -march=rv32i -mabi=ilp32 |
| crt0.S | Initializes stack to 0x0001_00FC, calls main(), traps on return |
| link.ld | ROM: 0x0–0x7FFF (32KB), SRAM: 0x10000–0x17FFF (8KB) |
| MMIO pattern | volatile pointers to fixed addresses for hardware access |
| rom.hex | 4-byte-wide Intel-compatible hex; loaded by $readmemh in sim |
| Firmware size | ~167 bytes (42 instructions) — extremely compact |
| Functionality | Print “Hello Deepak from Nielit!” × 10, then echo received bytes |
| Verification | make soc → TEST PASSED (PING echoed) |

Understanding firmware is essential for any SoC verification engineer. The hex values you see on cpu_mem_rdata during instruction fetch are the RISC-V opcodes directly compiled from this C code.

Previous: Sub-module 6 — AXI + UART Integration
Next: Sub-module 8 — Full SoC Integration